PDP-10 Simulator Usage

19-Feb-2016

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2016, written by Robert M Supnik

Copyright (c) 1993-2016, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	PDP-10 Features	3

2.1	CPU	4

2.2	Pager	6

2.3	Unibus Adapters	6

2.4	Front End (FE)	6

2.5	Timer (TIM)	7

2.6	PC11 Paper Tape Reader (PTR)	7

2.7	PC11 Paper Tape Punch (PTP)	8

2.8	DZ11 Terminal Multiplexer (DZ)	8

2.9	RH11 Adapter, RP04/05/06/07, RM02/03/05/80 drives (RP)	10

2.10	RH11 Adapter, TM02 Formatter, TU45 Magnetic Tape (TU)	11

2.11	LP20 DMA Line Printer (LP20)	12

2.12	RX211/RX02 Floppy Disk (RY)	12

2.13	CD20 Card Reader (CR)	13

3	Symbolic Display and Input	15

This memorandum documents the DEC PDP-10 simulator.

# Simulator Files

To compile the PDP-10, you must define VM_PDP10 and USE_INT64 as part of the compilation command line.

sim/		scp.h

sim_console.h

sim_defs.h

sim_ether.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_ether.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/pdp10/	pdp10_defs.h

pdp10_cpu.c

pdp10_fe.c

pdp10_ksio.c

pdp10_lp20.c

pdp10_mdfp.c

pdp10_pag.c

pdp10_rp.c

pdp10_sys.c

pdp10_tu.c

pdp10_xtnd.c

sim/pdp11/	pdp11_cr_dat.h

pdp11_xu.h

pdp11_cr.c

pdp11_dz.c

pdp11_pt.c

pdp11_ry.c

pdp11_xu.c

# PDP-10 Features

The PDP-10 simulator is configured as follows:

device name(s)	simulates

CPU			KS10 CPU with 1MW of memory

PAG			paging unit (translation maps)

UBA			Unibus adapters (translation maps)

FE			console

TIM			timer

PTR,PTP		PC11 paper tape reader/punch

RY			RX211/RX02 floppy disk and two drives

DZ			DZ11 8-line terminal multiplexer (up to 4)

LP20			LP20 line printer

CR			CD20 (CD11) card reader

RP			RH11 controller with eight RP04/RP05/RP06/RP07,

RM03/RM05/RM80 drives

TU			RH11/TM02 controller with eight TU45 drives

XU			DEUNA/DELUA Ethernet controller

The PTR, PTP, RX211, and CR are initially set DISABLED.  The DZ11 and LP20 can also be set DISABLED.  Some devices support the SET <device> ADDRESS command, which allows the I/O page address of the device to be changed, and the SET <device> VECTOR command, which allows the vector of the device to be changed.  All devices support the SHOW <device> ADDRESS and SHOW <device> VECTOR commands, which display the device address and vector, respectively.

The PDP-10 simulator implements several unique stop condition:

- Illegal instruction (000) in kernel mode

- Indirect addressing nesting exceeds limit (if enabled)

- Execute chaining exceeds limit (if enabled)

- Page fail or other error in interrupt sequence

- Illegal instruction in interrupt sequence

- Invalid vector pointer in interrupt sequence

- Invalid Unibus adapter number

- Non-existent exec or user page table address

The LOAD command supports RIM10B format paper tapes, SAV binary files, and EXE binary files.  LOAD switches -r, -s, -e specify RIM10, SAV, EXE format, respectively.  If no switch is specified, the LOAD command checks the file extension; .RIM, .SAV, .EXE specify RIM10, SAV, EXE format, respectively.  If no switch is specified, and no extension matches, the LOAD command checks the file format to try to determine the file type.

## CPU

The CPU options allow the user to specify the operating system to be run.  This in turn controls the microcode feature set, how system idling is detected, and how the system timer runs.

SET CPU TOPS10			TOPS-10

SET CPU TOPS20			TOPS-20

SET CPU ITS				ITS

SET CPU KLAD			diagnostic environment (no idle detection)

The CPU implements a SHOW command to display the I/O space address map:

SHOW CPU IOSPACE			show I/O space address map

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			18		program counter

FLAGS			18		processor flags (<13:17> unused)

AC0..AC17		36		active register set

IR			36		instruction register

EBR			18		executive base register

PGON			1		paging enabled flag

T20P			1		TOPS-20 paging

UBR			18		user base register

CURAC			3		current AC block

PRVAC			3		previous AC block

SPT			36		shared pointer table

CST			36		core status table

PUR			36		process update register

CSTM			36		CST mask

HSB			18		halt status block address

DBR1			18		descriptor base register 1 (ITS)

DBR2			18		descriptor base register 2 (ITS)

DBR3			18		descriptor base register 3 (ITS)

DBR4			18		descriptor base register 4 (ITS)

PIENB			7		PI levels enabled

PIACT			7		PI levels active

PIPRQ			7		PI levels with program requests

PIIOQ			7		PI levels with IO requests

PIAPR			7		PI levels with APR requests

APRENB		8		APR flags enabled

APRFLG		8		APR flags active

APRLVL		3		PI level for APR interrupt

IND_MAX		8		indirect address nesting limit;

if 0, no limit

XCT_MAX		8		execute chaining limit; if 0, no limit

PCQ[0:63]		18		PC prior to last jump or interrupt;

most recent PC change first

WRU			8		interrupt character

REG[0:127]		36		register sets

The CPU attempts to detect when the simulator is idle.  When idle, the simulator does not use any resources on the host system.  Idle detection is controlled by the SET IDLE and SET NOIDLE commands:

SET CPU IDLE			enable idle detection

SET CPU NOIDLE			disable idle detection

Idle detection is disabled by default and is operating system dependent:

TOPS-10				SOJG 6,1 in AC1 in user mode

TOPS-20				SOJG 2,3 in AC3 in monitor mode

ITS					AOJA 0,17 in AC17 in user mode

There is no idle detection in diagnostic mode.

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65536 entries.

## Pager

The pager contains the page maps for executive and user mode.  The executive page map is the memory space for unit 0, the user page map the memory space for unit 1.  A page map entry is 32 bits wide and has the following format:

bit			content

31			page is writeable

30			entry is valid

29:19			mbz

18:9			physical page base address

8:0			mbz

The pager has no registers.

## Unibus Adapters

The Unibus adapters link the system I/O devices to the CPU.  Unibus adapter 1 (UBA1) is unit 0, and Unibus adapter 3 is unit 1.  The adapter's Unibus map is the memory space of the corresponding unit.

The Unibus adapter has the following registers:

name			size		comments

INTREQ		32		interrupt requests

UB1CS			16		Unibus adapter 1 control/status

UB3CS			16		Unibus adapter 3 control/status

## Front End (FE)

The front end is the system console.  The keyboard input is unit 0, the console output is unit 1.  It supports one option:

SET FE STOP				halt the PDP-10 operating system

The front end has the following registers:

name			size		comments

IBUF			8		input buffer

ICOUNT		32		count of input characters

ITIME			24		input polling interval (if 0, the keyboard

is polled synchronously with the clock)

OBUF			8		output buffer

OCOUNT		32		count of output characters

OTIME			24		console output response time

## Timer (TIM)

The timer (TIM) implements the system timer, the interval timer, and the time of day clock used to get the date and time at system startup.  Because most PDP-10 software is not Y2K compliant, the timer implements one option:

SET TIM NOY2K			software not Y2K compliant, limit time

of day clock to 1999 (default)

SET TIM Y2K				software is Y2K compliant

The timer has the following registers:

name			size		comments

TIMBASE		59		time base (double precision)

TTG			36		time to go (remaining time) for interval

PERIOD		36		reset value for interval

QUANT			36		quantum timer (ITS only)

TIME			24		tick delay

Unless the CPU is set to diagnostic mode, the timer autocalibrates; the tick delay is adjusted up or down so that the time base tracks actual elapsed time.  This may cause time-dependent diagnostics to report errors.

## PC11 Paper Tape Reader (PTR)

The paper tape reader (PTR) reads data from a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper tape reader requires an unsupported driver under TOPS-10 and is not supported under TOPS-20 or ITS.

The paper tape reader implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

BUSY			1		busy flag (CSR<11>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	     	  STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

## PC11 Paper Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.   Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch requires an unsupported driver under TOPS-10 and is not supported under TOPS-20 or ITS.

The paper tape punch implements these registers:

name			size		comments

BUF			8		last data item processed

CSR			16		control/status register

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

DONE			1		device done flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

## DZ11 Terminal Multiplexer (DZ)

The DZ11 is an 8-line terminal multiplexer.  Up to 4 DZ11's (32 lines) are supported.  The number of lines can be changed with the command

SET DZ LINES=n			set line count to n

The line count must be a multiple of 8, with a maximum of 32.

The DZ11 supports three character processing modes, 7P, 7B, and 8B:

mode	input characters		output characters

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default is 7B, for compatibility with TOPS-20.

The DZ11 supports logging on a per-line basis.  The command

SET DZ LOG=line=filename

enables logging for the specified line to the indicated file.  The command

SET DZ NOLOG=line

disables logging for the specified line and closes any open log file.  Finally, the command

SHOW DZ LOG

displays logging information for all DZ lines.

The terminal lines perform input and output through Telnet sessions connected to a user-specified port.  The ATTACH command specifies the port to be used:

ATTACH {-am} DZ <port>		set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.  The optional switch -m turns on the DZ11's modem controls; the optional switch -a turns on active disconnects (disconnect session if computer clears Data Terminal Ready).  Without modem control, the DZ behaves as though terminals were directly connected; disconnecting the Telnet session does not cause any operating system-visible change in line status.

Once the DZ is attached and the simulator is running, the DZ will listen for connections on the specified port.  It assumes that the incoming connections are Telnet connections.  The connection remains open until disconnected by the simulated program, the Telnet client, a SET DZ DISCONNECT command, or a DETACH DZ command.

Other special DZ commands:

SHOW DZ CONNECTIONS 		show current connections

SHOW DZ STATISTICS 		show statistics for active connections

SET DZ DISCONNECT=linenumber	disconnects the specified line.

The DZ11 implements these registers:

name			size		comments

CSR[0:3]		16		control/status register, boards 0 to 3

RBUF[0:3]		16		receive buffer, boards 0 to 3

LPR[0:3]		16		line parameter register, boards 0 to 3

TCR[0:3]		16		transmission control register, boards 0 to 3

MSR[0:3]		16		modem status register, boards 0 to 3

TDR[0:3]		16		transmit data register, boards 0 to 3

SAENB[0:3]		1		silo alarm enabled, boards 0 to 3

RXINT			4		receive interrupts, boards 3 to 0

TXINT			4		transmit interrupts, boards 3 to 0

MDMTCL		1		modem control enabled

AUTODS		1		autodisconnect enabled

The DZ11 does not support save and restore.  All open connections are lost when the simulator shuts down or the DZ is detached.

## RH11 Adapter, RP04/05/06/07, RM02/03/05/80 drives (RP)

The RP controller implements the Massbus 18b (RH11) direct interface for large disk drives.  RP options include the ability to set units write enabled or write locked, to set the drive type to one of six disk types, or autosize:

SET RPn LOCKED			set unit n write locked

SET RPn WRITEENABLED		set unit n write enabled

SET RPn RM03			set type to RM03 (same as RM02)

SET RPn RM05			set type to RM05

SET RPn RM80			set type to RM80

SET RPn RP04			set type to RP04 (same as RP05)

SET RPn RP06			set type to RP06

SET RPn RP07			set type to RP07

SET RPn AUTOSIZE			set type based on file size at attach

SET RPn BADBLOCK			write bad block table on last track

The type options can be used only when a unit is not attached to a file.  Note that TOPS-10 V7.03 supported only the RP06 and RM03; V7.04 added support for the RP07.  TOPS-20 V4.1 also supported only the RP06 and RM03.  Units can be set ENABLED or DISABLED.  The RP controller supports the BOOT command.

The RP controller implements these registers:

name			size		comments

RPCS1			16		control/status 1

RPWC			16		word count

RPBA			16		bus address

RPCS2			16		control/status 2

RPDB			16		data buffer

RPDA[0:7]		16		desired surface, sector

RPDS[0:7]		16		drive status, drives 0 to 7

RPER1[0:7]		16		drive errors, drives 0 to 7

RPHR[0:7]		16		holding register, drives 0 to 7

RPOF[0:7]		16		offset, drives 0 to 7

RPDC[0:7]		8		desired cylinder, drives 0 to 7

RPER2[0:7]		16		error status 2, drives 0 to 7

RPER3[0:7]		16		error status 3, drives 0 to 7

RPEC1[0:7]		16		ECC syndrome 1, drives 0 to 7

RPEC2[0:7]		16		ECC syndrome 2, drives 0 to 7

RPMR[0:7]		16		maintenance register, drives 0 to 7

RPMR2[0:7]		16		maintenance register 2, drives 0 to 7

IFF			1		transfer complete interrupt request flop

INT			1		interrupt pending flag

SC			1		special condition (CSR1<15>)

DONE			1		device done flag (CSR1<7>)

IE			1		interrupt enable flag (CSR1<6>)

STIME			24		seek time, per cylinder

RTIME			24		rotational delay

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

end of file		x		assume rest of disk is zero

OS I/O error	x		report error and stop

## RH11 Adapter, TM02 Formatter, TU45 Magnetic Tape (TU)

The magnetic tape simulator simulates an RH11 Massbus adapter with one TM02 formatter and up to eight TU45 drives.  Magnetic tape options include the ability to make units write enabled or locked.

SET TUn LOCKED			set unit n write locked

SET TUn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET TUn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW TUn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.  The TU controller supports the BOOT command.

The magnetic tape controller implements these registers:

name			size		comments

MTCS1			16		control/status 1

MTBA			16		memory address

MTWC			16		word count

MTFC			16		frame count

MTCS2			16		control/status 2

MTFS			16		formatter status

MTER			16		error status

MTCC			16		check character

MTDB			16		data buffer

MTMR			16		maintenance register

MTTC			16		tape control register

INT			1		interrupt pending flag

DONE			1		device done flag

IE			1		interrupt enable flag

STOP_IOE		1		stop on I/O error

TIME			24		delay

UST[0:7]		16		unit status, units 0 to 7

POS[0:7]		32		position, units 0 to 7

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		operation incomplete

OS I/O error	parity error; if STOP_IOE, stop

## LP20 DMA Line Printer (LP20)

The LP20 is a DMA-based line printer controller.  There is one line printer option to clear the vertical forms unit (VFU):

SET LP20 VFUCLEAR			clear the vertical forms unit

The LP20 implements these registers:

name			size		comments

LPCSA			16		control/status register A

LPCSB			16		control/status register B

LPBA			16		bus address register

LPBC			12		byte count register

LPPAGC		12		page count register

LPRDAT		12		RAM data register

LPCBUF		8		character buffer register

LPCOLC		8		column counter register

LPPDAT		8		printer data register

LPCSUM		8		checksum register

DVPTR			7		vertical forms unit pointer

DVLNT			7		vertical forms unit length

INT			1		interrupt request

ERR			1		error flag

DONE			1		done flag

IE			1		interrupt enable flag

POS			32		position in output file

TIME			24		response time

STOP_IOE		1		stop on I/O error

TXRAM[0:255]	12		translation RAM

DAVFU[0:142]	12		vertical forms unit array

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

## RX211/RX02 Floppy Disk (RY)

RX211 options include the ability to set units write enabled or write locked, single or double density, or autosized:

SET RYn LOCKED			set unit n write locked

SET RYn WRITEENABLED		set unit n write enabled

SET RYn SINGLE			set unit n single density

SET RYn DOUBLE			set unit n double density (default)

SET RYn AUTOSIZE			set unit n autosized

The floppy disk requires an unsupported driver under TOPS-10 and is not supported under TOPS-20 or ITS.

The RX211 implements these registers:

name			size		comments

RYCS			16		status

RYBA			16		buffer address

RYWC			8		word count

RYDB			16		data buffer

RYES			12		error status

RYERR			8		error code

RYTA			8		current track

RYSA			8		current sector

STAPTR		4		controller state

INT			1		interrupt pending flag

ERR			1		error flag (CSR<15>)

TR			1		transfer ready flag (CSR<7>)

IE			1		interrupt enable flag (CSR<6>)

DONE			1		device done flag (CSR<5>)

CTIME			24		command completion time

STIME			24		seek time, per track

XTIME			24		transfer ready delay

STOP_IOE		1		stop on I/O error

SBUF[0:255]		8		sector buffer array

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

RX02 data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## CD20 Card Reader (CR)

The card reader (CR) implements a single controller (CD20, aka CD11) and card reader (e.g., Documation M200, GDI Model 100) by reading a file and presenting lines or cards to the simulator.  Card decks may be represented by plain text ASCII files, card image files, or column binary files.

Card image files are a file format designed by Douglas W. Jones at the University of Iowa to support the interchange of card deck data.  These files have a much richer information carrying capacity than plain ASCII files.  Card Image files can contain such interchange information as card-stock color, corner cuts, special artwork, as well as the binary punch data representing all 12 columns.  Complete details on the format, as well as sample code, are available at Prof. Jones's site: http://www.cs.uiowa.edu/~jones/cards/.

The card reader supports ASCII, card image, and column binary format card “decks.”  When reading plain ASCII files, lines longer than 80 characters are silently truncated.  Card image support is included for 80 column Hollerith, 82 column Hollerith (silently ignoring columns 0 and 81), and 40 column Hollerith (mark-sense) cards.  Column binary supports 80 column card images only.  All files are attached read-only (as if the -R switch were given).

ATTACH –A CR <file>			file is ASCII text

ATTACH –B CR <file>			file is column binary

ATTACH –I CR <file>			file is card image format

If no flags are given, the file extension is evaluated.  If the filename ends in .TXT, the file is treated as ASCII text.  If the filename ends in .CBN, the file is treated as column binary.  Otherwise, the CR driver looks for a card image header.  If a correct header is found the file is treated as card image format, otherwise it is treated as ASCII text.

The correct character translation MUST be set if a plain text file is to be used for card deck input.  The correct translation SHOULD be set to allow correct ASCII debugging of a card image or column binary input deck.  Depending upon the operating system in use, how it was generated, and how the card data will be read and used, the translation must be set correctly so that the proper character set is used by the driver.  Use the following command to explicitly set the correct translation:

SET TRANSLATION={DEFAULT|026|026FTN|029|EBCDIC}

This command should be given after a deck is attached to the simulator.  The mappings above are completely described at http://www.cs.uiowa.edu/~jones/cards/codes.html.  Note that DEC typically used 029 or 026FTN mappings.

DEC operating systems used a variety of methods to determine the end of a deck (recognizing that 'hopper empty' does not necessarily mean the end of a deck.  Below is a summary of the various operating system conventions for signaling end of deck:

RT-11:	12-11-0-1-6-7-8-9 punch in column 1

RSTS/E:	12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

RSX:		12-11-0-1-6-7-8-9 punch

VMS:		12-11-0-1-6-7-8-9 punch in first 8 columns

TOPS:		12-11-0-1 or 12-11-0-1-6-7-8-9 punch in column 1

Using the AUTOEOF setting, the card reader can be set to automatically generate an EOF card consisting of the 12-11-0-1-6-7-8-9 punch in columns 1-8.  When set to CD11 mode, this switch also enables automatic setting of the EOF bit in the controller after the EOF card has been processed.  [The CR11 does not have a similar capability.]  By default AUTOEOF is enabled.

SET CR AUTOEOF

SET CR NOAUTOEOF

The default card reader rate for the CD11 is 1000 cpm.  The reader rate can be set to its default value or to anywhere in the range 200..1200 cpm.  This rate may be changed while the unit is attached.

SET CR RATE={DEFAULT|200..1200}

It is standard operating procedure for operators to load a card deck and press the momentary action RESET button to clear any error conditions and alert the processor that a deck is available to read.  Use the following command to simulate pressing the card reader RESET button,

SET CR RESET

Another common control of physical card readers is the STOP button.  An operator could use this button to finish the read operation for the current card and terminate reading a deck early.  Use the following command to simulate pressing the card reader STOP button.

SET CR STOP

The simulator does not support the BOOT command.  The simulator does not stop on file I/O errors.  Instead the controller signals a reader check to the CPU.

The CR controller implements these registers:

name			size		comments

BUF			8		ASCII value of last column processed

CRS			16		CR11 status register

CRB1			16		CR11 12-bit Hollerith character

CRB2			16		CR11 8-bit compressed character

CRM			16		CR11 maintenance register

CDST			16		CD11 control/status register

CDCC			16		CD11 column count

CDBA			16		CD11 current bus address

CDDB			16		CD11 data buffer, 2nd status

BLOWER		2		blower state value

INT			1		interrupt pending flag

ERR			1		error flag (CRS<15>)

IE			1		interrupt enable flag (CRS<6>)

POS			32		file position - do not alter

TIME			24		delay time between columns

The CD11 simulation includes the Rev. J modification to make the CDDB act as a second status register during non-data transfer periods.

# Symbolic Display and Input

The PDP-10 simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as ASCII character

-c			display as six sixbit packed characters

-p			display as five packed ASCII (7b) characters

-m			display instruction mnemonics

-v			interpret address as virtual

-e			force executive mode

-u			force user mode

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		six sixbit packed characters

# or -p		five packed ASCII (7b) characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses standard PDP-10 assembler syntax.  There are three instruction classes: memory reference, memory reference with AC, and I/O.

Memory reference instructions have the format

memref {@}address{(index)}

memory reference with AC instructions have the format

memac ac,{@}address{(index)}

and I/O instructions have the format

io device,{@}address{(index)}

where @ signifies indirect.  The address is a signed octal number in the range 0 - 0777777.  The ac and index are unsigned octal numbers in the range 0-17.  The device is either a recognized device mnemonic (APR, PI, TIM) or an octal number in the range 0 - 0177.

The simulator recognizes the standard MACRO alternate mnemonics (CLEAR for SETZ, OR for IORI), the individual definitions for JRST and JFCL variants, and the extended instruction mnemonics.